Installation Guide:

ALT-YD-BE6004-LED-(All)

Tools Needed:

- ¼” ratchet

- Panel popper

- 8mm socket

- Remove (4x) 8mm bolts securing tail light.

- Remove (2x) plastic retainers securing trunk liner panel using the panel popper. Remove liner and set it aside.

- Unseat and remove tail light.

- Disconnect wiring harness.

- Transfer revers bulb to Spyder tail light socket with Blk/Wht wire. (Never touch bulbs with bare hands)

- Remove bulb plate by squeezing release tabs.

- Transfer rear fog lamp bulb to Spyder tail light socket with Blu/Blk wire.

- Transfer inner parking/brake lamp bulb to Spyder tail light socket with Blk/Brwn wire.

- Seat Spyder tail light.

9. Locate gasket and apply it to Spyder tail light.

12.Connect wiring harness to Spyder tail light.

11.Reinstall (4x) 8mm nuts.

14. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

13.Replace trunk liner panel and install (2x) retainers.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.