Installation Guide:

ALT-YD-CS07V2-LED-(All)

Tools Needed:

- Phillips Screwdriver

- Unseat tail light.

- Remove (2x) phillips screws.

- Remove top (brake light) and bottom (brake light) bulbs from sockets.

- Remove (3x) bulb sockets from tail light. (Never touch bulbs with bare hands)

- Install top brake light socket plug into Spyder tail light. (Making sure black wire on socket plug lines up with black wire on socket)

- Install included weather proof black cap onto lower brake light socket. (Not used with Spyder tail light)

- Install reverse light socket into Spyder tail light.

7. Install brake light socket into Spyder tail light.

- Reinstall (2x) phillips screws securing tail light.

- Seat Spyder tail light.

11. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.