Installation Guide:

ALT-YD-DMAG05-LED-(All)

Tools Needed:

- Panel Popper

- Remove tail light access cover.

- Remove (2x) plastic retainers using the panel popper.

- Unseat tail light.

- Remove (1x) thumbscrew securing the tail light.

- Remove bulb sockets from tail light, next remove harness from tail light. (Make sure never to touch bulbs with bare hands)

- Disconnect tail light harness.

8. Install reverse socket and turn signal socket into Spyder tail light.

7. Remove brake lamp bulb from socket.

10. Connect wiring harness to Spyder tail light.

9. Install the socket insert into the brake light socket making sure to line up the black wire on the socket to the black wire on the socket insert.

12. Reinstall (2x) plastic retainers.

11. Seat Spyder tail light.

14. Reinstall access cover.

13. Reinstall thumbscrew securing tail light.

15. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.