Installation Guide:

ALT-YD-FM05-LED-BK

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Inside trunk remove (1x) plastic retainer.

- Remove (2x) plastic retainers with panel popper.

- Remove trunk liner panel.

- Remove thumb screw securing panel.

- Remove (3x) 10mm nuts securing tail light.

- Unseat tail light.

- Remove reverse light socket.

- Remove (3x) brake bulb sockets.

- Install reverse light socket into Spyder tail light.

- Remove (3x) brake light bulbs from sockets.

- Install (3x) socket insert into brake bulb sockets. (Align black wire of socket with black wire of socket insert)

- Install (3x) sockets into Spyder tail light.

- Reinstall (3x) 10mm nuts.

- Seat Spyder tail light.

- Reinstall plastic retainer.

- Reinstall trunk panel and thumbscrew.

18. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

- Reinstall (2x) plastic clips.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.