Installation Guide:

ALT-YD-FM05V3-LED-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Inside trunk remove (1x) plastic retainer.

- Remove (2x) plastic retainers with panel popper.

- Remove trunk liner panel.

- Remove thumb screw securing panel.

- Remove (3x) 10mm nuts securing tail light.

- Disconnect harness and push through rubber grommet.

- Remove reverse light bulb and transfer to Spyder tail light. (Never touch bulb with bare hands)

- Unseat tail light.

- Install reverse light socket into Spyder tail light.

- Pull Spyder tail light harness threw hole and seat rubber grommet.

- Seat Spyder tail light.

- Connect Spyder tail light harness.

- Reinstall (3x) 10mm nuts.

- Reinstall trunk panel and thumbscrew.

- Reinstall (2x) plastic clips.

- Reinstall plastic retainer.

18. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.