Installation Guide:

ALT-YD-FM99-LED-BK

Tools Needed:

- ¼” ratchet

- 10mm socket

- 8mmm socket

- Remove trunk liner.

- Remove (1x) thumbscrew

- Unseat tail light.

- Remove (4x) 10mm nuts.

- Remove (4x) bulb sockets from tail light.

- Remove outer brake bulb from socket.

- Remove center light bulb and transfer to Spyder tail light. (Never touch bulb with bare hands)

- Install outer socket insert into brake socket. (Align black wire of socket to black wire of insert)

- Install center socket insert into brake socket. (Align black wire of socket to black wire of insert)

- Repeat procedure for inner brake light and insert all bulb sockets and secure.

- Seat Spyder tail light.

- Install (4x) 8mm nuts.

- Reinstall thumbscrew.

- Reinstall carpet liner.

15.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.