Installation Guide:

ALT-YD-HC06-2D-LED-(All)

Tools Needed:

- ¼” ratchet

- T30 Torx

- 10mm socket

- Panel popper

- Phillips screw driver

- Remove (2x) plastic retainers.

- Open trunk and remove (2x) T30 torx screws.

- Remove center trunk panel.

- Remove (4x) phillips screws that secure the bumper.

- Unseat and remove bumper fascia.

- Remove (2x) 10mm nuts.

- Remove (4x) 10mm buts securing the tail light.

- Remove (3x) plastic retainers securing the trunk liner.

10.Unseat tail light.

9. Disconnect tail light harnesses and remove turn signal and brake light socket.

12.Seat Spyder tail light.

11.Remove (1x) phillips screw securing tail light bracket. Transfer bracket to Spyder tail light.

14.Remove brake light bulb from socket.

13.Reinstall (4x) 10mm nuts securing tail light.

16.Install turn signal and brake sockets into Spyder tail light. Install (2x) harness plugs into Spyder tail light.

15.Install Spyder brake socket insert into brake socket. (Making sure to align black wire of socket insert to black wire of socket)

18.Install (2x) 10mm nuts.

17.Reinstall trunk liner and (3x) plastic retainers.

20.Reinstall (4x) phillips screws.

19.Seat bumper fascia.

21.Reinstall (2x) T30 Torx

22.Reinstall (2x) plastic retainers.

23. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.