Installation Guide:

ALT-YD-LGS98-LED-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Phillips screw driver

- Remove (5x) plastic retainers securing trunk panel.

- Peel back trunk panel and carpet.

- Remove (2x) 10mm nuts securing outer tail light.

- Remove wire harness retainer clip and disconnect harness.

- Remove (2x) plastic covers.

- Push rubber grommet threw to remove outer tail light.

- Unseat outer tail light.

- Remove (2x) 10mm screws securing tail light.

- Remove decklid liner.

- Moving to inner tail light. Remove (14x) retainers securing decklid liner.

12.Remove (2x) 10mm nuts securing outer tail light.

11.Disconnect inner tail light harness.

- Locate a 3156a bulb and put in turn signal socket on Spyder tail light. (Never touch bulb with bare hands)

13.Unseat inner tail light.

16.Locate included resister pack and affix to inner deck lid and tighten (2x) phillips screws to secure resister pack.

15.Pull inner tail light harness threw trunk and seat Spyder inner tail light.

18.Install (2x) 10mm nuts securing inner tail light.

17.Connect resister pack harness and Spyder inner tail light harness.

20.Feed wiring harness threw body and seat Spyder inner tail light.

19.Reinstall decklid liner and install (14x) plastic retainers.

21.Reinstall (2x) 10mm nuts securing Spyder tail light. Connect Spyder harness.

22.Reinstall trunk liner and panel using (5x) plastic retainers.

23.Reinstall (2x) 10mm screws.

24.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.