Installation Guide:

ALT-YD-MITEVO1008-LED-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Remove (2x) 10mm bolts.

- Remove (2x) plastic retainers.

- Pull back trunk liner to access outer tail light harness.

- Disconnect outer tail light harness.

- Remove (7x) plastic retainers on trunk lid.

- Unseat outer tail light.

- Remove (2x) 10mm nuts securing inner tail light.

- Remove trunk lid liner.

- Unseat inner tail light.

- Disconnect inner tail light harness.

12. Unseat tail light socket and remove bulb. Transfer socket to Spyder tail light and install socket insert.

11. Transfer revers light bulb and socket to Spyder inner tail light.

14. Remove (3x) bulb sockets from outer tail light.

13. Peel backing from weather seal and stick to Spyder trail light.

16. Remove brake bulb and replace with Spyder tail light bulb insert. (Align black wire of harness to black wire of socket insert)

15. Replace turn signal bulb with provided amber bulb. (Never touch bulb with bare hands)

18. Seat outer Spyder tail light.

17. Seat brake, signal and side marker sockets into Spyder tail light.

20. Connect outer tail light harness.

19. Reinstall (2x) 10mm bolts.

22. Seat Spyder inner tail light.

21. Reinstall trunk liner and (2x) plastic retaining screws.

24. Reinstall (2x) 10mm nuts.

23. Connect inner tail light harnesses.

26. Reinstall (7x) plastic retainers.

25. Align and seat trunk lid liner.

27.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.