Installation Guide:

ALT-YD-NGTR09-LED-BK

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Remove (2x) clips securing wheel well liners.

- Remove (3x) 10mm bolts securing each side of bumper.

- Disconnect tail light harness.

- Remove (4x) plastic clips securing each side of trunk liner.

- Remove weather strip.

- Remove (5x) 10mm nuts securing the bumper.

- Unseat grommet and pull out wire harness.

- Unseat rear bumper to gain access to rubber grommet.

- Remove (4x) 10mm bolts securing tail light.

- Remove bumper.

12. Reinstall (4x) 10mm bolts securing tail light.

11. Seat Spyder tail light.

14. Seat bumper.

13. Route tail light harness and seat rubber grommet.

15. Connect tail light harness.

16. Reinstall brackets and (3x) 10mm bolts.

18. Reinstall (5x) 10mm nuts securing the bumper.

17. Seat liner and install (2x) plastic clips.

20. Reinstall weather strip.

19. Reinstall trunk liner and (4x) plastic retainers.

21.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.