Installation Guide:

ALT-YD-SI085D-SEQ-(All)

Tools Needed:

- ¼” ratchet

- 8mm and 10mm socket

- Panel popper

- Phillips screw driver

- Remove cargo mat.

- Remove (2x) retaining clips securing spare tire foams.

- Remove (2x) plastic clips securing rear panel.

- Remove cover to reveal bolt securing side panel.

- Remove (1x) 10mm bolt.

- Remove (3x) phillips screws.

- Remove (2x) 10mm bolt.

- Unseat tail light.

- Connect Spyder tail light harness.

- Disconnect harness.

- Push in Spyder wires and seat new rubber grommet.

- Push factory harness grommet in.

- Seat Spyder tail light.

- Tuck in remaining wires. (Note there will be (2x) unused wires for US models)

- Remove (3x) plastic clips.

- Reinstall (2x) 10mm bolts.

- Route harness up threw loosened panels.

- Pull back plastic panels to access new wires for inner tail light.

.

- Route harness threw rubber boot.

- Loosen top and bottom of rubber boot.

- Pull off (4x) trunk panel to route inner tail light harness.

- Push wire up and reinstall rubber boot.

.

- Remove (3x) 8mm nuts securing inner tail light.

- Remove inner tail light socket.

.

- Unseat inner tail light.

- Seat Spyder inner tail light.

.

- Route new Spyder harness following factory wires and connect (2x) wire harnesses to inner tail light. (Align black wire of socket to black wire of socket insert)

- Reinstall (3x) 8mm nuts securing inner tail light.

.

- Push in wires and route remaining green wire to license plate light.

- Tap green wire to factory black wire with white stripe. Reinstall License plate socket.

.

- Realign plastic panels and install (3x) phillips screws.

- Reinstall (4x) hatch plastic trims.

.

- Reinstall rear trunk panel and (2x) plastic clips.

- Reinstall (1x) 10mm bolt and plastic cover.

.

- Reinstall (3x) plastic clips.

- Reinstall spare tire foams and (2x) retaining clips.

.

- Reinstall security shade.

- Reinstall trunk liner.

.

38.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.