Installation Guide:

ALT-YD-TCAM07-LED-(All)

Tools Needed:

- ¼” ratchet

- 8, 10 and 12mm socket

- Panel popper

- Remove (1x) 8mm grocery hook bolt.

- Unseat & Remove trunk panel.

- Remove (3x) 10mm nuts.

- Disconnect tail light harness.

- Remove fascia cap.

- Remove (1x) 10mm bolt.

- Remove (2x) 10mm bolts.

- Remove (1x) 10mm bolt.

- Remove (3x) 10mm bolts and (2x) 12mm nuts

- Remove (4x) 10mm bolts.

- Unseat tail light.

- Unseat bumper fascia.

- Remove decklid liner.

- Remove (13x) plastic retainers.

- Remove (3x) 10mm nuts.

- Disconnect inner tail light harness.

- Remove reverse bulb from OEM tail light. (Never touch bulbs with bare hands)

- Unseat inner tail light.

.

- Remove fascia clip from OEM tail light.

- Install bulb into Spyder inner tail light and reinstall socket.

- Install fascia clip onto Spyder tail light.

- Remove brake and turn signal socket from OEM tail light.

.

- Remove brake bulb from socket and install socket insert. (Align socket insert black wire to black wire of socket)

- Install turn signal into Spyder tail light.

.

- Seat Spyder tail light.

- Reinstall (3x) 10mm nuts.

.

- Seat Spyder inner tail light.

- Reconnect harness.

.

- Reinstall (3x) 10mm nuts.

- Reconnect wire harness.

.

- Reinstall decklid liner.

- Reinstall trunk panel and grocery hook (1x) 8mm bolt.

.

- Seat rear bumper fascia.

- Reinstall (13x) plastic retainers.

.

- Reinstall (1x) 10mm bolt.

- Reinstall fascia (1x) 10mm bolt and cap.

.

- Reinstall underbody panel and reinstall (7x) 10mm bolts and (2x) 12mm nuts.

- Reinstall (2x) 10mm bolts.

.

38.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.