Installation Guide:

ALT-YD-VG15-LED-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Open trunk liner door.

- Remove (1) thumb screw securing outer tail light.

- Disconnect wiring harness.

- Unseat tail light.

5. Connect Spyder outer tail light.

6. Seat Spyder outer tail light.

- Open inner tail light access panel.

- Install thumb screw and close carpet door.

- Remove (2x) 10mm nut securing inner tail light.

- Disconnect inner tail light harness.

- Seat Spyder inner tail light.

- Unseat inner tail light.

14. Reinstall (2x) 10mm nuts.

13. Connect inner tail light harness.

16. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder LED tail lights.

15. Tuck wires in and close access panel.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.