Installation Guide:

PRO-YD-AI94-HL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips screwdriver

- Remove (1x) phillips screw.

- Unseat corner lamp.

- Remove turn signal sockets.

- Remove (9x) phillips screws.

- Remove (4x) 10mm bolts.

- Remove (1x) phillips screw on each side of fascia.

- Remove (2x) 10mm bolts.

- Remove (2x) phillips screws on the underside of the fascia.

- Remove (4x) 10mm bolts securing headlight.

- Unseat and remove front fascia.

- Remove high and low beam bulbs and remove headlight.

- Unseat headlight.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Remove high beam bulb from socket.

- Seat Spyder headlight.

- Connect high beam harness.

- Reinstall (4x) 10mm bolts securing headlight.

- Insert low beam bulb.

.

- Reinstall support bracket and (9x) phillips screws.

- Seat front fascia.

- Reinstall (1x) phillips screw on each side of the front fascia.

- Reinstall (2x) 10mm bolts on each side of the front fascia.

- Seat corner lamp.

- Reinstall turn signal sockets into corner lamp.

.

- Reinstall (1x) corner screw securing corner lamp.

.

26.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.