Installation Guide:

PRO-YD-BMWE4602-4D-AM-CCFL-(All)

Tools Needed:

- ¼” ratchet

- 8mm socket

- Panel popper

- Gently slide bumper molding outwards.

- Remove center molding.

- Remove side moldings by releasing tabs.

- Remove (6x) plastic retainers using panel popper.

- Remove (2x) 8mm bolts under the front fascia on each corner.

- Under center of the front fascia remove (3x) plastic retainers.

- Remove both outer lower grills.

- Under headlight remove garnish by pushing in tab and gently pull out.

- Unseat fascia from ductwork.

- Remove front bumper fascia.

- Remove (2x) upper 8mm bolts.

- Remove (2x) 8mm hidden lower headlight bolts.

- Unseat corner lamp.

- Remove (1x) phillips screw securing corner lamp.

- Unseat headlight.

- Disconnect corner lamp harness.

- Remove low beam bulb from OEM headlight (Never touch bulbs with bare hands).

- Disconnect high and low beam harness.

.

- Make sure to reinstall low beam seal securely.

- Install low beam bulb into Spyder headlight.

- Reconnect the (3x) headlight harnesses.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Seat Spyder headlight.

- Black and White wire plug is onlu used in euro market applications.

.

26.Reinstall headlight garnish.

- Reinstall upper and lower 8mm bolts.

.

28.Reinstall both outer lower grills.

- Seat front bumper fascia.

.

30.Reinstall center molding.

29.Reinstall (6x) plastic retainers.

.

32.Reinstall (2x) corner 8mm bolts on each side.

31.Reinstall both corner moldings.

.

34.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

33.Reinstall (3x) plastic retainers.

.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.