Installation Guide:

PRO-YD-BMWE9005V2-AM-BK

Tools Needed:

- ¼” ratchet

- 8mm and 10mm socket

- Phillips screwdriver

- T30 Torx

- Remove (4x) T30 torx.

- Remove (7x) phillpis screws from below the front bumper fascia.

- Remove (2x) 8mm screws on each side of the fascia.

- Remove (2x) phillips screws behind fender liner on each side.

- Unseat front bumper fascia.

- Disconnect both fog lamps.

- Remove (2x) T30 torx screw.

- Remove (2x) 10mm headlight bolts.

- Unseat headlight.

- Remove (1x) T30 torx screw from behind the headlight corner.

- Remove (2x) 10mm bolts securing headlight bracket to headlight.

- Disconnect headlight harness.

- Connect Spyder headlight harness.

- Install bracket onto Spyder headlight using (2x) 10mm bolts.

- Install (2x) 10mm bolt on top of headlight.

- Seat Spyder headlight.

- Reinstall (1x) T30 torx screw from behind the corner of headlight.

- Reinstall (2x) T30 torx screws.

.

- Seat bumper fascia.

- Reinstall both fog lamp bulbs.

- Reinstall (2x) screws on each side securing bumper.

- Reinstall (4x) T30 torx.

- Reinstall (7x) screw on the bottom of the bumper fascia.

- Reinstall (2x) 8mm screws securing fender liner on each side.

.

25.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.