Installation Guide:

PRO-YD-BMWE9208-DRL-(All)

Tools Needed:

- ¼” ratchet

- 8mm and 10mm socket

- Phillips screwdriver

- T30 and T20 Torx

- Remove (7x) 8mm screws below the front bumper fascia.

- Remove (2x) 8mm screws on the passenger side inner fender.

- Remove (3x) 8mm screws along the edge of the inner fender well.

- Remove access panel by turning tabs 90 degrees counter clockwise.

- Remove (2x) 8mm screw on each side of the bumper.

- Remove (2x) 10mm nuts on the driver side inner fender.

- Unseat the front bumper fascia (Check all accessories/fog lights and disconnect prior).

- Remove (4x) T30 torx bolts securing the bumper.

- Remove 5th T30 torx screw pointing upwards that is hidden behind the upper corner of the headlight.

- Remove (4x) T30 torx screws securing the headlight.

- Unseat headlight assembly.

- 6th and last headlight screw is located below the headlight.

- Remove and transfer over mounting bracket to Spyder headlight by removing (2x) T30 torx bolts.

- Disconnect headlight harness.

- Transfer over AFS unit to Spyder headlight there are (4x) T20 torx screws securing it.

- Transfer over OEM HID ballast to Spyder headlight there are (4x) T20 torx screw securing it.

- Remove plastic cover and remove OEM HID bulb and AFS motor.

- Secure HID ballast and AFS unit using OEM hardware.

.

- Install Spyder cover using supplied hardware.

- Install HID bulb and AFS motor into Spyder headlight (Never touch bulb with bare hands).

- Connect Spyder headlight harness.

- Connect corner light wiring.

- Reinstall (6x) headlight screws securing the headlight.

- Seat Spyder headlight.

.

- Reinstall (2x) 8mm fender screws.

- Seat the front bumper fascia.

.

- Reinstall (7x) 8mm screws.

- Reinstall (4x) T30 torx screws.

.

- Reinstall all screw and nuts securing fender liner.

- Reinstall fender liner access panels.

.

31.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.