Installation Guide:

PRO-YD-C305-CCFL-(All)

Tools Needed:

- ¼” ratchet

- 7mm and 10mm socket

- Panel popper

- Remove (4x) plastic retainers in the wheel well.

- Remove (4x) plastic retainers on top of grill.

- Pull back fender liner and remove (3x) 10mm bolts.

- Remove (1x) plastic retainer securing bumper fascia.

- Remove (6x) 7mm bolts below the bumper fascia.

- Remove (2x) 7mm bolts on corner of bumper fascia.

- Remove bumper fascia.

- Disconnect side markers.

- Remove (3x) 10mm bolts securing headlight.

- Unseat headlight.

- Remove turn signal socket from OEM headlight (Never touch bulbs with bare hands).

- Disconnect low beam, high beam and turn signal harness.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Install turn signal socket onto Spyder headlight.

- Seat Spyder headlight.

- Connect Spyder headlight low beam, high beam and turn signal harness.

- Seat bumper fascia.

- Reinstall (3) 10mm bolts securing Spyder headlight.

.

- Reinstall (3x) 10mm bolts.

- Reinstall plastic retainer.

- Reinstall (6x) 7mm bolts.

- Reinstall (4x) plastic retainers.

- Reinstall (4x) plastic retainers.

- Reinstall (2x) 7mm bolts on corners.

.

25.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.