Installation Guide:

PRO-YD-CCAM2010-CCFL-(All)

Tools Needed:

- ¼” ratchet

- 7mm and 10mm socket

- T15 Torx

- Panel popper

- Remove (6x) plastic retainers on the top edge of the bumper fascia.

- Remove (2x) 10mm bolts.

- Moving below remove (2x) 10mm bolts.

- Remove (3x) T15 torx screws that secure the fender liner.

- Remove (3x) 10mm bolts behind the fender liner.

- Remove (1x) 7mm bolt securing edge of bumper fascia.

- Unseat bumper and disconnect fog light and corner lights. Remove bumper fascia and set aside.

- Remove (1x) 10mm bolt hidden behind this shaft.

- Remove (2x) lower 7mm bolts securing the headlight.

- Remove (2x) upper 7mm bolts securing the headlight.

- Disconnect headlight and turn signal harness.

- Unseat the headlight.

- Remove turn signal socket from OEM headlight (Never touch bulbs with bare hands).

- Install turn signal socket into Spyder headlight.

15.For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Connect Spyder headlight and turn signal harness.

- Install (2x) lower 7mm bolts.

- Seat Spyder headlight.

.

- Reconnect fog light and corner light harnesses.

- Install (2x) upper 7mm bolts.

- Reinstall the (3x) 10mm bolts.

- Seat the front bumper fascia.

- Reinstall (1x) 7mm bolts securing the bumper fascia corner.

- Reinstall (1x) 10mm bolt hidden behind the shaft.

.

26.Reinstall (2x) 10mm bolts below the bumper fascia.

- Reinstall fender liner and install (3x) plastic retainers.

.

28.Reinstall (2x) edge 10mm bolts.

- Reinstall (6x) plastic retainers.

.

29.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.