Installation Guide:

PRO-YD-CCK88-(All)

Tools Needed:

- ¼” ratchet

- 7mm and 10mm socket

- Phillips screwdriver

- Remove (5x) 7mm bolts securing the grill.

- Remove (4x) phillips screws securing bumper light and signal.

- Unseat light and remove (2x) light sockets.

- Locate this tab behind the light securing the grill. Pull up on tab to release grill.

- Unseat the grill and disconnect coroner lights.

- Locate and remove (4x) 10mm bolts securing headlight.

- Remove high beam bulb from OEM headlight (Never touch bulbs with bare hands).

- Disconnect high and low beam harnesses.

- Connect low beam harness and route wire under the bracket. Connect high beam bulb harness.

- Install high beam bulb onto Spyder headlight.

- Reinstall corner light harness and seat the grill.

- Seat Spyder head light and install (4x) 10mm bolts.

- Install turn signal bulbs onto turn signal housing.

- Make sure to carefully pull the bumper signal light bulbs threw the opening.

- Reinstall (5x) 7mm bolts securing the grill.

- Seat turn signal and reinstall (4x) phillips screws.

17.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.