Installation Guide:

PRO-YD-CCO15-LBDRL-(All)

Tools Needed:

- ¼” ratchet

- 7, 8 and 10mm socket

- T15 Torx

- Panel popper

- Remove (6x) T15 torx screws securing the grill.

- Remove (3x) T15 torx screws securing the fender liner.

- Pull back fender liner and remove (3x) 7mm bolts securing the bumper fascia.

- Remove (6x) 8mm bolts below the bumper fascia.

- Remove (2x) 10mm bolts holding the bumper in place.

- Unseat bumper fascia.

- Remove (4x) 7mm bolts securing the headlight.

- Disconnect fog light harness and set bumper aside.

- Disconnect headlight harness.

- Unseat headlight.

- Disconnect headlight and turn signal harness.

- Connect Spyder headlight harness.

- Reconnect fog light harness.

- Reinstall (4x) 7mm bolts securing the headlight

- Reinstall (2x) 10mm bolts.

15.Seat front bumper fascia.

- Install (3x) T15 torx screws securing fender liner.

- Reinstall (3x) 7mm bolts on corner of bumper.

.

- Reinstall (6x) 8mm bolts securing bottom of bumper fascia.

- Reinstall (6x) T15 screws securing grill.

21.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.