Installation Guide:

PRO-YD-CE02-DRL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel popper

- Remove (8x) plastic retainers securing radiator support cover.

- Remove radiator support cover.

- Remove (1x) 10mm bolt securing the grill.

- Release (2x) retainer clips securing the grill.

- Unseat the grill.

- Remove (2x) headlight rod pins.

- Unseat the headlight.

- Disconnect headlight harness.

- Seat Spyder headlight.

- Connect Spyder headlight harness.

- Reinstall grill and clip in (2x) grill retainer clips.

- Reinstall (2x) headlight pins.

- Seat radiator support cover.

- Reinstall (1x) 10mm bolt securing the grill.

16.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

15.Reinstall (8x) plastic retainers.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.