Installation Guide:

PRO-YD-CHIP14-LB-(All)

Tools Needed:

- ¼” ratchet

- 7, 10mm socket

- T25, T20, T15 Torx

- Panel popper

- Remove (10x) plastic retainers securing the radiator shroud.

- Remove radiator shroud.

- Remove (2x) T25 torx screws.

- Remove (8x) T20 torx screws.

- Remove (1x) 7mm bolt screw.

- Remove (4x) T15 screws securing the fender liner.

- Remove (1x) 10mm bolt securing the bumper fascia to the fender.

- Unseat front bumper fascia.

- Remove (1x) 7mm bolt on side of headlight.

- Under headlight remove (1x) 7mm bolt.

- Unseat headlight.

- Remove (2x) 7mm bolts on top of headlight.

- Connect Spyder headlight harness.

- Disconnect wire harness.

- Reinstall (4x) 7mm bolts securing the headlight.

- Seat Spyder headlight.

- Reinstall (1x) 10mm bolt securing bumper to fender.

17.Seat front bumper fascia.

- Reinstall (4x) T15 torx screws.

- Reinstall (1x) 7mm bolts.

- Reinstall (8x) T20 screws.

- Reinstall (2x) T25 torx bolts.

- Reinstall (10x) plastic retainers.

- Reinstall radiator shroud.

25.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.