Installation Guide:

PRO-YD-DMAG05-CCFL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- T30 Torx

- Panel popper

- Remove (6x) plastic retainers securing the header panel.

- Remove (3x) plastic retainers securing the fender liner.

- Pull back liner and remove (2x) 10mm bolts.

- Remove (1x) plastic retainer that sits behind the bolts you just removed.

- Remove (2x) T30 torx screws that secure the headlight.

- Unseat bumper fascia making sure to disconnect fog light wire harness.

- Unseat the headlight.

- Disconnect the low beam, high beam and turn signal harnesses.

- Install OEM turn signal harness onto Spyder headlight.

- Remove OEM turn signal harness from OEM headlight.

- Connect low beam, high beam and turn signal harness on Spyder headlight.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Seat Spyder headlight.

- Reinstall (2x) T30 torx screws.

- Reinstall (6x) plastic retaining clips along the header panel.

- Seat the front bumper fascia making sure to reconnect fog lights.

- Reinstall (1x) plastic retainer.

17.Reinstall (2x) 10mm bolts securing the bumper fascia.

20.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Reinstall (3x) plastic retainers.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.