Installation Guide:

PRO-YD-FF15018-LB-(All)

Tools Needed:

- ¼” ratchet

- 7, 8 and 10mm socket

- Panel popper

- T25 Torx

- Using panel popper remove (14x) plastic retainers securing the radiator shroud.

- Remove radiator shroud and set it aside.

- Disconnect (2x) sensor clips connected to grill.

- Remove (4x) 10mm bolts securing the grill.

- Remove bumper trim.

- Remove (2x) 7mm bolts securing bumper trim on each side.

- Remove grill and set aside.

- Remove (2x) 7mm bolts securing grill.

- Remove (1x) plastic retainer and (2x) 10mm bolts securing the headlight.

- Remove (1x) 8mm and (3x) 10mm bolts securing the headlight.

- Disconnect (2x) headlight clips and remove turn signal socket.

- Unseat the headlight.

- Remove (3x) T25 torx and transfer bracket to Spyder headlight.

- Connect (2x) headlight clips and install turn signal socket onto Spyder headlight.

- Reinstall (1x) plastic retainer and (2x) 10mm bolts securing the headlight.

- Seat Spyder headlight.

- Reinstall grill securing it with (2x) lower 8mm bolts and (4x) upper 10mm bolts.

- Reinstall (1x) 8mm and (3x) 10mm bolts securing the bottom of the Spyder headlight.

- Reinstall (2x) sensor slips onto grill.

- Reinstall bumper trim and (2x) 7mm bolts securing each side.

22.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Reinstall radiator shroud and (14x) retaining slips.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.