Installation Guide:

PRO-YD-P98705-DRL-(All)

Tools Needed:

- Phillip’s Screwdriver

- Unlock and open factory storage compartment door to locate factory tools.

- Get the factory hex tool.

- Pull back liner to access headlight release access hole.

Unlock plastic inner fender liner screw.

- Unseat and remove headlight.

- Insert factory hex tool, twist to release headlight.

- Release (3x) clips securing back cover.

- Remove (4x) Philips screws from back of factory headlight.

- Remove (5x) Philips screw from behind Spyder headlight.

- Remove factory headlight bulb for install onto Spyder headlight.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Install bulb onto Spyder headlight and reinstall (5x) Philips screw and cover.

- Using hex tool lock headlight into place.

- Seat Spyder headlight.

- Reinstall fender liner and lock fender liner screw.

- Return factory tool box to storage compartment.

18.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Close factory tool box compartment and secure.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.