Installation Guide:

PRO-YD-SWRX08-HID-LBDRL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips Screwdriver

- Panel popper

- Remove (2x) plastic fasteners holding the intake duct with panel popper.

- Remove intake duct.

- Remove (3x) plastic fasteners securing the inner fender liner.

- Remove (4x) plastic fasteners using panel popper.

- Below the bumper fascia remove (6x) fasteners securing the bumper.

- Carefully pull-on edges of bumper fascia to unclip and remove bumper fascia.

- Remove (2x) 10mm bolts securing outside corner of headlight.

- Unplug fog light harness.

- Reinstall (2x) 7mm bolts.

- Remove (2x) 10mm bolts on inside corner of headlight.

- Unseat headlight.

- Remove (1x) 10mm bolt and (1x) plastic clip securing top of headlight.

- Remove factory headlight ballast by removing (3x) Philips screws and unclipping harness from inside.

- Unplug (4x) harnesses from rear of headlight.

- Remove AFS motor by unscrewing all the way out using 10mm socket.

- Remove headlight cover from rear of headlight.

- Remove grounding strap by pulling it off the connector.

- Remove (1x) Philips screw holding ground strap to bulb holder. Twist and remove harness from bulb.

- Unscrew and remove low beam cover from rear of Spyder headlight.

- Remove manual headlight adjuster from Spyder headlight.

- Install HID ignitor harness onto Spyder headlight.

- Carefully remove HID bulb from factory headlight and install onto Spyder headlight.

- Install AFS motor and low beam cover onto Spyder headlight.

- Install HID ballast onto Spyder headlight.

- Seat Spyder headlight and reconnect (4x) headlight harnesses.

- Remove lower bracket from OEM headlight and install onto Spyder headlight.

- Seat Spyder headlight and reconnect (4x) headlight harnesses.

- Reinstall (1x) 10mm bolt and (1x) plastic clip securing top of headlight.

- Install (2x) inner 10mm bolts securing the headlight.

- Install (2x) outer 10mm bolts securing the headlight.

- Reinstall intake duct.

- Reinstall front bumper fascia and fog light harnesses.

- Reinstall (6x) retainers securing low part of bumper fascia.

- Reinstall (6x) plastic retainers securing the bumper and intake duct.

35.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.