Installation Guide:

PRO-YD-TCAM07-DRL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips Screwdriver

- Panel popper

- Remove (2x) Phillips screws.

- Remove (1x) 10mm bolts.

- Remove (1x) plastic fasteners securing the fender liner.

- Remove (6x) 10mm bolts from below the bumper fascia. (Not all shown)

- Unseat bumper fascia. (If equipped with fog lights disconnect harnesses)

- Pull back fender liner and remove (1x) 10mm bolt securing corner of bumper fascia.

- Remove (3x) 10mm bolts securing the headlight.

- Unseat headlight.

- Remove turn signal and side marker socket.

- Disconnect low beam and high beam harness.

- Install turn signal and side marker socket onto Spyder headlight.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Seat Spyder headlight.

- Connect low beam and high beam harness onto Spyder headlight.

- Seat front bumper fascia. (If equipped reconnect fog lights)

- Reinstall (3x) 10mm bolts securing Spyder headlight.

- Reinstall (1x) 10mm bolt.

- Reinstall (2x) Philips screws.

- Reinstall (1x) 10mm bolt securing corner of bumper fascia.

- Replace fender liner and (1x) plastic retainer.

22. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Replace (6x) 10mm bolts securing bottom of bumper fascia.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.