Installation Guide:

PRO-YD-TCAM15-LB-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Panel Popper

- Remove (9x) push-type plastic retainers.

- Remove radiator support cover.

- Remove (1x) pop-up plastic retainer from middle of the bumper fascia.

- Remove (2x) pop-up retainers from inner bumper fascia corners.

- Remove (2x) plastic retainer near the fender liner securing the corners of the bumper fascia.

- Remove (1x) 10mm bolts on inner passenger and driver side headlight area.

- Carefully unseat both corners of the bumper fascia.

- Remove (1x) 10mm bolt on either side.

- Unseat headlight.

- Remove (2x) upper 10mm bolts securing the headlight.

- Disconnect low beam and high beam harness.

- Remove side marker light bulb.

- Install Spyder bulb socket onto headlight harness. (Making sure to align black wire of socket to black wire of harness) Install socket onto Spyder headlight.

- Connect low beam and high beam harness onto Spyder headlight.

- Seat Spyder headlight.

16.Reinstall all 10mm bolts securing the headlight.

- Seat both corners of bumper fascia.

- Reinstall (2x) plastic retainers on either side.

20. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Reinstall all plastic retainers and radiator support cover.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.