Installation Guide:

PRO-YD-THLAN11-3DDRL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips Screwdriver

- Panel popper

- Remove (11x) push-type plastic retainers.

- Remove filler panel.

- Remove grill.

- Remove (6x) Philips plastic retainers.

- Remove (2x) plastic retainers near fender liner.

- Remove (3x) plastic retainers.

- Remove (4x) 10mm bolts securing bottom of bumper fascia.

- Remove (1x) 10mm bolts securing corner of bumper fascia.

- Unseat and remove bumper fascia. (If equipped disconnect fog light harnesses)

- Remove (2x) 10mm bolts on bottom corners of bumper fascia.

- Unseat headlight.

- Remove (3x) 10mm bolts securing headlight.

- Remove turn signal socket from OEM headlight and install onto Spyder headlight. (Never touch bulbs with bare hands)

- Disconnect low beam, high beam and side marker harnesses.

- Connect low beam, high beam and turn signal harness onto Spyder headlight.

- For info on how to wire DRL’s please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Reinstall (3x) 10mm bolts securing the Spyder headlight.

- Seat Spyder headlight.

- Seat bumper fascia. (If equipped reinstall fog light harnesses)

- Reinstall (3x) plastic retainers.

- Seat filler panel and reinstall (11x) plastic push-type retainers.

- Seat grill and install (6x) Philips plastic retainers.

- Reinstall (4x) 10mm bolts below bumper fascia.

- Reinstall (2x) plastic retainer and (1x) 10mm bolts near fender liner.

26. Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

- Reinstall (2x) 10mm bolts on either side of bumper fascia.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.