Installation Guide:

PRO-YD-TT05V2-LB-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips Screwdriver

- Panel popper

- Remove (2x) push-type plastic retainers.

- Remove (2x) Philips screw.

- Remove (1x) plastic retainer.

- Remove grill.

- Remove (4x) plastic retainers securing bottom of bumper fascia.

- Remove (3x) Philips screws securing fender liner.

- Unseat bumper fascia. (If equipped disconnect fog light harnesses)

- Remove (3x) plastic retainers securing top of bumper fascia.

- Remove headlight garnish.

- Remove (4x) 10mm bolts securing the headlight.

- Disconnect headlight, side marker and turn signal harnesses.

- Unseat the headlight.

- Remove turn signal socket and side marker socket from OEM headlight and install onto Spyder headlight.

- Disconnect low beam, high beam and side marker harnesses.

- Connect headlight, side marker and turn signal harness onto Spyder headlight.

- Remove bulb from side marker and install Spyder socket insert.

- Reinstall (4x) 10mm bolts securing the Spyder headlight.

- Seat Spyder headlight.

- Reinstall headlight garnish and (1x) retaining clip.

- Seat front bumper fascia. (Reconnect fogs light harnesses)

- Seat grill and reinstall (2x) plastic retainers and (2x) Philips screws securing the grill.

- Reinstall (3x) plastic retainers.

- Reinstall (4x) plastic retainers securing the bumper fascia.

- Reinstall (3x) Phillips screws.

25.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.