Installation Guide:

PRO-YD-TTC08-HL-(All)

Tools Needed:

- ¼” ratchet

- 10mm socket

- Phillips Screwdriver

- Panel popper

- Remove (5x) plastic retainers using the panel popper.

- Remove (2x) 10mm bolts below the bumper fascia.

- Remove (1x) 10mm bolt on either corner of the bumper fascia.

- Remove (1x) plastic retainer securing lower corners of bumper fascia.

- Remove (3x) 10mm bolts securing the headlight.

- Unseat the bumper fascia.

- Unseat headlight.

- Remove (1x) Philips screw.

- Remove bumper bracket below headlight and install onto Spyder headlight.

- Disconnect low beam and high beam harnesses. Remove turn signal socket.

- Install low beam, high beam and side marker harnesses onto Spyder headlight.

- Remove city light and side marker harness and install onto Spyder headlight.

- Seat Spyder headlight.

- Install turn signal socket and harness onto Spyder headlight.

- Reinstall (3x) 10mm bolts and (1x) Phillips screw securing Spyder headlight.

- For info on how to wire halos please refer to the Spyder FAQ playlist on the Spyder Auto YouTube channel.

- Reinstall (3x) plastic retainers. Note you will not use the 2 smaller retainers anymore.

- Seat bumper fascia.

- Reinstall (1x) 10mm bolts on either corner of the bumper fascia.

- Reinstall (2x) 10mm bolts and (1x) plastic retainer on either corner of the lower bumper fascia.

21.Turn on lights and confirm all functions are working properly. You have successfully completed the install of your new Spyder Headlights.

For further help with installation, please watch our installation video. Follow the direct link below to our YouTube channel for more information:

http://www.youtube.com/watch

DISCLAIMER

Spyder Auto Installation Guide is not liable for damages, injuries, and death while installing the product. This is intended to be an installation guide, to reduce installation time. We have no responsibility for improper installation. If you do not know how to install, please have a professional install it.